EXPERIMENT 2

DATE :

Determination and plotting of Electric field and Electric flux density in sphere full of charges with varying radius.

Aim:

To write program for electric field and electric flux density in sphere using MATLAB.

Software required:

MATLAB (R2018 or later)

Program:

Test case (i) Determination and plotting of electric field and electric flux density in sphere full of charges with varying radius

clc;

clear;

q=input('Enter the value of q=');

r=input('Enter the value of r=');

epsilon=8.854e-12;

k=1/(4*pi*epsilon);

E=k*q/(r^2);

disp("The electric field is");

disp(E);

D=epsilon*E;

disp("The flux density is");

disp(D);

% Plot

x=linspace(0.1,10,50);

E_plot=k*q./(x.^2);

D_plot=epsilon*E_plot;

figure;

subplot(1,2,1);

plot(x,E_plot);

xlabel('Radius');

ylabel('Electric field');

subplot(1,2,2);

plot(x,D_plot);

xlabel('Radius');

ylabel('Flux density')

Output:-

Enter the value of q=2*10^-2

Enter the value of r=0.1*10^-3

Test case (ii) Derive and plot the electric field (E) and electric flux density (D) inside a  uniformly charged sphere with charge density ρ₀

Program:

clc;

rho0 = 1e-6;

epsilon = 8.854e-12;

R = 1;

r = linspace(0, R, 100);

D = (rho0 .* r) / 3;

E = D / epsilon;

subplot(1,2,1);

plot(r,D,'LineWidth',2);

xlabel("Radius (m)");

ylabel("Electric Flux Density (D)");

title("Uniform Density: D vs Radius");

subplot(1,2,2);

plot(r,E,'LineWidth',2);

xlabel("Radius (m)");

ylabel("Electric Field (E)");

title("Uniform Density: E vs Radius");

Output: Linear increase in D and Linear increase in E

Test case (iii) For a sphere with a linearly increasing charge density ρᵥ = ρ₀r, derive and plot  the electric field (E) and electric flux density (D)

Program:

clc;

rho0 = 1e-6;

epsilon = 8.854e-12;

R = 1;

r = linspace(0, R, 100);

D = (rho0 .* r.^2) / 4;

E = D / epsilon;

subplot(1,2,1);

plot(r,D,'LineWidth',2);

xlabel("Radius (m)");

ylabel("Electric Flux Density (D)");

title("Linear Density: D vs Radius");

subplot(1,2,2);

plot(r,E,'LineWidth',2);

xlabel("Radius (m)");

ylabel("Electric Field (E)");

title("Linear Density: E vs Radius");

Output:

Test case (iv) A sphere of radius R = 2 m has a uniform charge density ρ = 3×10⁻⁶ C/m³.  Calculate and plot electric field inside and outside the sphere

Program:

clc;

rho0 = 3e-6;

epsilon = 8.854e-12;

R = 2;

r = linspace(0.1, 4, 100);

E = zeros(size(r));

for i = 1:length(r)

if r(i) < R

E(i) = rho0 * r(i) / (3 * epsilon);

else

Q = rho0 * (4/3) * pi * R^3;

k = 1/(4*pi*epsilon);

E(i) = k * Q / (r(i)^2);

end

end

plot(r,E,'LineWidth',2);

xlabel("Radius (m)");

ylabel("Electric Field (E)");

title("Electric Field Inside and Outside Sphere");

Output:-

• Inside: Linear increase

• Outside: 1/r² decay

| Parameters q = 2×10⁻² C, r = 1×10⁻⁴ m ρ₀ = 1×10⁻⁶ C/m³, r = 1 m (inside sphere) ρᵥ = ρ₀r, r = 1 m ρ = 3×10⁻⁶ C/m³, R = 2 m (r = 1 m, inside) ρ = 3×10⁻⁶ C/m³, R = 2 m (r = 3 m, outside) | Observed/Simulated Result | Theoretical Result | Error Difference ΔE = 5 × 10¹² ΔD = 1 × 10² ΔD ≈ 0 ΔE ≈ 0 ΔD ≈ 0 ΔE ≈ 0 ΔE ≈ 0 ΔE ≈ 0 |
| --- | --- | --- | --- |
|  | E = 1.7975 × 10¹⁶ V/m D = 1.592 × 10⁵ C/m² | E = 1.797 × 10¹⁶ V/m D = 1.591 × 10⁵ C/m² |  |
|  | D = 3.33 × 10⁻⁷ C/m² E = 3.76 × 10⁴ V/m | D = 3.33 × 10⁻⁷ C/m² E = 3.76 × 10⁴ V/m |  |
|  | D = 2.5 × 10⁻⁷ C/m² E = 2.82 × 10⁴ V/m | D = 2.5 × 10⁻⁷ C/m² E = 2.82 × 10⁴ V/m |  |
|  | E = 1.13 × 10⁵ V/m | E = 1.13 × 10⁵ V/m |  |
|  | E = 5.65 × 10⁴ V/m | E = 5.65 × 10⁴ V/m |  |

Result :

Thus the program was verified for electric field and electric flux density in sphere using SCILAB was  successfully.