J.Priya

192512394

ECA5403-ELECTROMAGNETIC FILEDS AND WAVES

C01-1

EXPERIMENT 1

AIM:

To write a MATLAB program for transformation between Cartesian, Cylindrical and Spherical coordinate systems.

PROGRAM:

clc;

x=input('Enter the value of X=');

y=input('Enter the value of y=');

z=input('Enter the value of z=');

% Cartesian to Cylindrical

rho=sqrt(x^2+y^2);

phi=atan2d(y,x);

disp('Cartesian to cylindrical coordinate system of p(rho,phi,z)=');

disp([rho phi z]);

% Cartesian to Spherical

r=sqrt(x^2+y^2+z^2);

theta=acosd(z/r);

phi=atan2d(y,x);

disp('Cartesian to spherical coordinate system of s(r,theta,phi)=');

disp([r theta phi]);

clc;

rho=input('Enter the value of rho=');

phi=input('Enter the value of phi=');

z=input('Enter the value of z=');

% Cylindrical to Cartesian

x=rho*cosd(phi);

y=rho*sind(phi);

disp('Cylindrical to Cartesian coordinate system of p(x,y,z)=');

disp([x y z]);

% Cylindrical to Spherical

r=sqrt(rho^2+z^2);

theta=acosd(z/r);

disp('Cylindrical to spherical coordinate system of s(r,theta,phi)=');

disp([r theta phi]);

clc;

r=input('Enter the value of r=');

theta=input('Enter the value of theta (deg)=');

phi=input('Enter the value of phi (deg)=');

% Spherical to Cartesian

x=r*sind(theta)*cosd(phi);

y=r*sind(theta)*sind(phi);

z=r*cosd(theta);

disp('Spherical to Cartesian coordinate system of p(x,y,z)=');

disp([x y z]);

% Spherical to Cylindrical

rho=r*sind(theta);

disp('Spherical to cylindrical coordinate system of s(rho,phi,z)=');

disp([rho phi z]);

OUTPUT:

RESULTS:

Thus the transformation between Cartesian, Cylindrical and Spherical coordinate systems have been done using MATLAB software.

| S. No. | Objective | Assessment Criterion | Excellent – 4 | Good – 3 | Satisfactory – 2 | Needs Improvement – 1 | Weighting (%) | Marks given | Marks obtained |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Apply coordinate-system definitions and angle conventions | Apply coordinate-system definitions, symbols, units, and angle conventions | Applies coordinate-system definitions and angle conventions throughout. | Uses correct definitions with one minor notation or convention error. | Shows partial understanding; multiple corrections are needed. | Uses incorrect coordinate definitions or angle conventions. | 25 | 2.5 |  |
| 2 | Implement transformations in MATLAB | Implement six transformations in MATLAB | Implements all six transformations correctly using suitable degree-based functions. | Implements most transformations correctly with minor coding errors. | Implements some transformations but needs assistance to correct errors. | Unable to implement the required transformations. | 25 | 2.5 |  |
| 3 | Verify numerical values and reverse transformations | Verify numerical values and reverse transformations | All outputs agree with analytical values; reverse transformations recover the original point within rounding precision. | Most values agree; small unexplained differences occur. | Several values differ or reverse verification is incomplete. | Outputs are incorrect or not verified. | 30 | 3 |  |
| 4 | Interpret and document results | Interpret and document results | Presents complete tables, units, deviations, and a clear conclusion. | Documentation is clear with minor omissions. | Tables or interpretation are incomplete. | Results and conclusion are missing or unclear. | 20 | 2 |  |
| Total |  |  |  |  |  |  | 100 | 10 |  |